# DAM WATER LEVEL MONITORING SYSTEM USING ULTRASONIC SENSOR

# AIM

The aim of this project is to design and implement an automated Dam Water Level Monitoring System using an ultrasonic sensor and Arduino microcontroller to continuously monitor water levels and provide timely alerts during warning and danger conditions.

# OBJECTIVES

• To monitor water level without physical contact
• To display real-time distance and status on LCD
• To provide visual indication using LEDs
• To generate distinct audible alerts for warning and danger
• To improve safety and early flood warning

# COMPONENTS REQUIRED

1. Arduino Uno
2. Ultrasonic Sensor (HC-SR04)
3. I2C LCD (16×2)
4. Green, Yellow, Red LEDs
5. Buzzer
6. Resistors (330Ω)
7. Connecting Wires
8. Power Supply

# WORKING PRINCIPLE

The ultrasonic sensor emits ultrasonic waves toward the water surface. The waves reflect back and are received by the sensor. The time taken is measured and converted into distance using the speed of sound. Based on the measured distance, the system determines the water level condition.

# SYSTEM OPERATION

Safe Condition (>20 cm): Green LED ON, buzzer OFF
Warning Condition (10–20 cm): Yellow LED ON, warning buzzer for 3 seconds
Danger Condition (≤10 cm): Red LED ON, continuous danger alarm

# BLOCK DIAGRAM

Ultrasonic Sensor → Arduino → LCD Display
          ↓
             LEDs & Buzzer

# FLOWCHART

[PLACEHOLDER FOR FLOWCHART IMAGE]

Start → Trigger Sensor → Measure Distance → Compare Thresholds → Display Status → Alarm Output → Repeat

# CIRCUIT DIAGRAM

TRIG → Pin 9
ECHO → Pin 10
Green LED → Pin 4
Yellow LED → Pin 5
Red LED → Pin 6
Buzzer → Pin 7
LCD → SDA & SCL

# ADVANTAGES

• Non-contact measurement
• Low cost
• Real-time monitoring
• Easy to implement
• Expandable to IoT

# LIMITATIONS

• Affected by environmental conditions
• Limited sensing range
• Requires stable mounting

# APPLICATIONS

• Dam monitoring
• Flood warning systems
• Water tank monitoring
• Irrigation systems

# FUTURE SCOPE

• GSM-based SMS alerts
• IoT cloud monitoring
• Automatic gate control
• Data logging and analytics

# CONCLUSION

The project successfully demonstrates a reliable dam water level monitoring system using ultrasonic sensing. The use of visual and audible alerts enhances safety, and the system can be further upgraded for real-world deployment.